Automatic Water Tank Controller

Abstract

Water scarcity and improper water management are common problems in many residential and industrial areas. Manual monitoring of water tanks often leads to issues such as overflow, wastage of water, and damage to the pump due to dry running. To overcome these problems, an Automatic Water Tank Controller system is designed using Verilog Hardware Description Language.

The proposed system monitors the water level using multiple sensors and automatically controls the water pump. When the tank level falls below the minimum threshold, the controller activates the pump to fill the tank. When the tank becomes full, the pump automatically stops. The system also includes an alarm indication to notify the user when the tank is completely filled. The design is simulated using Vivado and verified through behavioral simulation.

This project demonstrates the application of digital logic design and hardware description language in implementing a real-time control system.

Introduction

Water tank controllers are widely used in homes, apartments, and industries to automate water management. Traditionally, water pumps are operated manually, which requires continuous monitoring of the tank water level. This manual operation can result in water overflow or pump damage due to dry running.

With advancements in digital electronics and VLSI technology, automated systems can be developed to perform these tasks efficiently. In this project, an automatic water tank controller is implemented using Verilog HDL and simulated in Vivado.

The system uses three water level sensors to detect the water level inside the tank. Based on the sensor signals, the controller decides whether the pump should be turned ON or OFF. This ensures efficient water management and prevents unnecessary wastage.

Objectives

The main objectives of this project are:

• To design an automatic water tank controller using Verilog HDL.
• To monitor water levels using digital sensor inputs.
• To automatically control the pump operation.
• To prevent water overflow when the tank is full.
• To avoid dry running of the pump when the tank is empty.
• To simulate the design using Vivado.
• To demonstrate the practical application of VLSI design in real-world automation systems.

System Components

1. Low Level Sensor

The low level sensor detects when the water level in the tank is below the minimum level. When this sensor is activated, the system turns ON the pump to start filling the tank.

2. Mid Level Sensor

The mid level sensor indicates that the tank has reached a medium water level. At this stage the pump continues to operate until the tank becomes full.

3. High Level Sensor

The high level sensor detects when the tank is completely filled. Once this sensor is triggered, the controller turns OFF the pump to prevent overflow.

4. Water Pump

The pump is responsible for pumping water into the tank from an external water source.

5. Alarm/Buzzer

An alarm signal is generated when the tank becomes full to notify the user.

6. Controller

The controller is designed using Verilog HDL and processes the sensor inputs to generate appropriate control signals.

Working Principle

The Automatic Water Tank Controller operates based on the signals received from the water level sensors.

Initially, when the tank is empty, the low level sensor becomes active. This signal is detected by the controller, which then turns ON the water pump to begin filling the tank.

As the water level increases and reaches the mid level sensor, the pump continues to run to fill the remaining portion of the tank.

When the water reaches the high level sensor, the controller detects that the tank is full and immediately turns OFF the pump to prevent overflow. At the same time, an alarm signal is generated to inform the user that the tank is filled.

If the water level decreases again below the low level sensor, the controller restarts the pump automatically.

Inputs and Outputs

Inputs

The system uses the following input signals:

Clock – Synchronizes the operation of the digital circuit.
Reset – Resets the system to its initial state.
Low Level Sensor – Detects low water level.
Mid Level Sensor – Detects medium water level.
High Level Sensor – Detects full water level.

Outputs

Pump Control Signal – Controls the ON/OFF operation of the pump.
Alarm Signal – Indicates when the tank is full.
Level Status – Shows the current water level condition.

Advantages

• Reduces water wastage by preventing overflow.
• Protects pump from dry running.
• Fully automatic operation.
• Reduces manual effort.
• Reliable digital control system.
• Easy to implement using FPGA and VLSI tools.
• Low cost and energy efficient.

Applications

• Residential water tank automation systems.
• Apartment water supply management.
• Industrial water storage tanks.
• Agricultural irrigation systems.
• Smart home automation systems.
• Municipal water supply control.

Conclusion

The Automatic Water Tank Controller is an efficient solution for managing water levels in storage tanks. By using digital logic design and Verilog HDL, the system can automatically control the pump operation based on the water level sensors.

Simulation results obtained in Vivado confirm the correct operation of the controller. The system successfully turns the pump ON when the water level is low and turns it OFF when the tank becomes full. This helps prevent water overflow and pump damage.

The proposed design demonstrates how VLSI technology can be applied to real-world automation problems. The system can be further extended by implementing it on FPGA hardware and integrating additional features such as wireless monitoring and IoT connectivity.

Simulation Input Values

| Time (ns) | Reset | Low Level | Mid Level | High Level | Pump (Expected) | Alarm |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 10 | 0 | 1 | 0 | 0 | 1 | 0 |
| 20 | 0 | 1 | 1 | 0 | 1 | 0 |
| 30 | 0 | 0 | 1 | 0 | 1 | 0 |
| 40 | 0 | 0 | 1 | 1 | 0 | 1 |
| 50 | 0 | 0 | 0 | 1 | 0 | 1 |
| 60 | 0 | 1 | 0 | 0 | 1 | 0 |

Explanation of Simulation Steps

Step 1 – System Reset

Time = 0 ns

- Reset = 1

- All sensors = 0

- Pump = OFF

The system initializes to a safe state.

Step 2 – Tank Empty

Time = 10 ns

- Low sensor = 1

- Pump turns ON to start filling the tank.

Step 3 – Tank Half Filled

Time = 20 ns

- Mid sensor = 1

- Pump continues running.

Step 4 – Water Rising

Time = 30 ns

- Mid level active

- Pump still ON.

Step 5 – Tank Full

Time = 40 ns

- High sensor = 1

- Pump turns OFF

- Alarm turns ON.

Step 6 – Tank Draining

Time = 60 ns

- High level becomes 0

- Low level becomes 1

- Pump starts again.

CODE

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 06.03.2026 18:11:36

// Design Name:

// Module Name: project_27

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

`timescale 1ns / 1ps

//=================================================

// Automatic Water Tank Controller (Behavioral)

//=================================================

module water_tank_controller(

input clk,

input reset,

input low_level,      // Low level sensor

input mid_level,      // Mid level sensor

input high_level,     // High level sensor

output reg pump,

output reg alarm,

output reg [2:0] level_status

);

// level_status

// 000 = Empty

// 001 = Low

// 010 = Medium

// 100 = Full

always @(posedge clk or posedge reset)

begin

if(reset)

begin

pump <= 0;

alarm <= 0;

level_status <= 3'b000;

end

else

begin

// Tank Empty

if(low_level == 1 && mid_level == 0 && high_level == 0)

begin

pump <= 1;

alarm <= 0;

level_status <= 3'b001;

end

// Tank Half Filled

else if(mid_level == 1 && high_level == 0)

begin

pump <= 1;

alarm <= 0;

level_status <= 3'b010;

end

// Tank Full

else if(high_level == 1)

begin

pump <= 0;

alarm <= 1;

level_status <= 3'b100;

end

// Default

else

begin

pump <= pump;

alarm <= 0;

end

end

end

endmodule

//=================================================

// Testbench

//=================================================

module water_tank_controller_tb;

reg clk;

reg reset;

reg low_level;

reg mid_level;

reg high_level;

wire pump;

wire alarm;

wire [2:0] level_status;

water_tank_controller uut(

.clk(clk),

.reset(reset),

.low_level(low_level),

.mid_level(mid_level),

.high_level(high_level),

.pump(pump),

.alarm(alarm),

.level_status(level_status)

);

// Clock generation

always #5 clk = ~clk;

initial

begin

clk = 0;

reset = 1;

low_level = 0;

mid_level = 0;

high_level = 0;

#10 reset = 0;

// Tank empty

#10 low_level = 1;

// Water rising

#20 mid_level = 1;

// Tank full

#20 high_level = 1;

// Tank draining

#20 high_level = 0;

mid_level = 0;

low_level = 1;

#30 $finish;

end

endmodule